Asterisk Version 1.4.15

Customer Configuration Guide (CCG)

For Use with

AT&T IP Flexible Reach

Issue 1.0

09/23/2008

Table of Contents

1.	Introduction	3

2.	Special Notes	3

2.1.	Emergency 911/E911 Services Limitations	3

2.2.	Asterisk Setup and Programming	3

2.3.	Asterisk Operational Caveats	4

3.	Certification Overview	6

4.	How to Prepare for IP Flex Reach Service Turn-up	8

4.1.	Information from Customer to AT&T	8

4.2.	Information to Customer from AT&T	9

5.	Configuration Guide	9

5.1.	Asterisk Version and Feature Requirements	10

5.2.	Asterisk Configuration Files	11

5.2.1	extensions.conf	12

5.2.2	rtp.conf	16

5.2.3	sip.conf	16

6.	Troubleshooting	19

A.	Appendix - PSTN / Primary Rate Interface (PRI) Overflow	22

A.1.	zaptel.conf	22

A.2.	zapata.conf	23

B.	Appendix - Other Asterisk Configuration Files	28

B.1.	indications.conf	28

B.2.	modules.conf	28

C.	Appendix – Telephone Configuration in sip.conf	30

D.	Appendix – Use of Other SIP Devices	32

E.	Appendix - G.711 and T.38 Analog Terminal Adapters	33

# Introduction

AT&T IP Flexible Reach provides SIP trunking of IP-PBXs. This document provides a configuration guide to assist Asterisk Version 1.4.15 administrators in connecting to AT&T IP Flexible Reach.

Traditional PSTN overflow as well as the individual station programming for Asterisk is included in the Appendices.

AT&T has tested Asterisk 1.4.15 with IP Flex Reach service in two network arrangements:

- An IP-PBX connecting to the PSTN for inbound and outbound traffic

- An IP-PBX as part of a private corporate network aka site to site.

This testing is functional in nature, typically one call at a time. No load testing was performed.

# Special Notes

## Emergency 911/E911 Services Limitations

While AT&T IP Flexible Reach services support E911/911 calling capabilities in certain circumstances, there are significant limitations on how these capabilities are delivered. Please review the AT&T IP Flexible Reach service Guide in detail to understand these limitations and restrictions.

Asterisk Version 1.4.15 will be referred to as “Asterisk” through the rest of the document.

## Asterisk Setup and Programming

AT&T does not provide general installation or configuration support for the Open Source versions of Asterisk.  Support for the Open Source versions of Asterisk may be available from a Digium reseller or directly from Digium for a fee.  For more information or configuration assistance, please contact a Digium reseller or call Digium Technical Support at +1 256 428 6000.

## Asterisk Operational Caveats

The Asterisk feature set does not include:

- G.729 Annex B (Voice Activity Detection, Comfort Noise) is not supported by Asterisk. The G.729A codec is supported and requires a license. The license can be purchased at www.digium.com .

AT&T’s certification testing of Asterisk has identified several operational caveats. These include:

- Sending video codecs in the SIP messaging will create call failures. Make sure that video is not enabled as part of the signaling to the AT&T network.

- When interacting with traditional voicemail, auto-attendant or similar application, repeated, rapid entry of an identical digit on a telephone keypad may result in lost digits. This caveat does NOT apply to operations such as dialing a telephone number to initiate a call.

- Ringback is not available on blind transfer scenarios. The call rings at the correct station and completes as desired but no ringback is heard by the party being transferred.

- The second station in a three-way calling arrangement, where one leg is an IP conferencing platform, will lose the ability to enter DTMF digits.

- T.38 Fax is not supported. T.38 testing produced mixed results. Successful T.38 fax transmission was very dependent on the specific Analog Telephone Adapter (ATA). (The most consistent performance was obtained with an ATA that is now manufacturer discontinued.) See the Appendix E for a summary of the tested ATAs.

- Fax calls from the ABE C.1.6.2 to a fax machine, where the destination fax machine is a super G3 fax machine and in the PSTN and in an AT&T Hosted Internet Protocol Communication Service (HIPCS) local serving area, will fail.

- Placing station-to-station calls via ten digit dialing (routed through the AT&T network) to the same Asterisk server requires the parameter “directrtpsetup = yes” in sip.conf. AT&T notes that the directrtpsetup parameter is considered experimental. (This option sets up the call using the originating device’s IP address so a re-INVITE is not invoked there.)

- Only SIP trunking was tested. The H.323 protocol was not tested. The AT&T network does not support the Inter-Asterisk eXchange IAX protocol.

- Use of the SIP method UPDATE is not supported. Do not set “canreinvite=update” in sip.conf.

- Load testing was NOT performed. AT&T certification only addresses functional testing.

# Certification Overview

This section provides a service overview of the Asterisk Version 1.4.15 integration with AT&T IP Flexible Reach.

Private Side

Legacy Circuit

PBX

AT&T Managed

Cisco router

With

Voice GW

Call Control

Element

Public Side

PSTN

ATA

Switch

Corporate Site #1

AT&T

Managed

Router

With NAT

IP Border

Element

Network

Gateway

Border

Element

PSTN

Application

Server

Customer Premises

Phones and server in private address space.

Firewall or Managed Router does NAT.

Customer sites connect to ATT IP Border Element

NFAS

PRI

Asterisk IP PBX

Corporate Site #2

The test Asterisk 1.4.15 customer premises consisted of the following components.

- Polycom 600 IP phones – These phones use the SIP protocol to communicate to the Asterisk IP PBX for call feature and routing support. These phones support Power over Ethernet (PoE) as well as Cisco in-line power and are  powered accordingly. These phones used firmware version 2.2.0.0047.

- Snom 360 IP phone – These phones use the SIP protocol to communicate to the Asterisk IP PBX for call feature and routing support. These phones support Power over Ethernet (PoE) and are powered accordingly. This phone used firmware version 6.5.10.

- Mediatrix 1102 – An ATA using the SIP protocol to communicate to the Asterisk IP PBX for call feature and routing support. This ATA provides an endpoint for T.38 fax but that feature was disabled. This ATA was using firmware version 5.0.22.138 E.

- Fax machine – These machines have an RJ-11 standard analog interface.  The Mediatrix ATA was tested as a SIP adaptor for the analog interface.

- Asterisk Version 1.4.15 – This is the central component of this IP-PBX.

  - Server – An IBM eServer running Linux CentOS 2.6.9-55.0.12.plus.c4.smp.

  - PRI Interface – Digium TE410 PCI card. Supporting 4 T1s, this board was configured in a Non Facility Associated Signaling (NFAS) Primary Rate Interface (PRI) mode. This T1 voice card is used to connect to the local PSTN. This interface was only tested as a backup/alternative route for PSTN connectivity.

  - Codec support included G.711 mu law and G.729A. (The G.729A codec is not included with Asterisk Version 1.4.15 and must be purchased separately.) G.729B (Voice Activity Detection (VAD) and Comfort Noise) are NOT supported by Asterisk.

- AT&T Managed Router – This is the router is provided and managed by AT&T.

The following routing scenarios are supported by the Asterisk IP PBX and DO NOT use the AT&T Call Control. These scenarios shall use the G.711 or G.729A codecs.

- Intra Asterisk IP PBX phone calls i.e. local phone to local phone

The following routing scenarios are supported by the Asterisk IP PBX and DO use the AT&T Call Control. For voice calls, the G.729A codec is the first choice with G.711 mu law being a second offering.

- Asterisk IP PBX phones to PSTN (domestic US and international).

- Asterisk IP PBX phones to legacy PBX site via a Cisco gateway.

- Legacy PBX site via a Cisco gateway to Asterisk IP PBX phones.

- Asterisk IP PBX phones at one Asterisk IP PBX IP PBX site to Asterisk IP PBX phones at another IP PBX site.

If the customer has subscribed to the AT&T IP Local service, then the following routing scenarios are supported by the Asterisk IP PBX IP PBX and DO use the AT&T Call Control. For voice calls, the G.729A codec is the first choice with G.711 mu law being a second offering.

- Inbound PSTN to Asterisk IP PBX phone

- Outbound local PSTN calls from the Asterisk IP PBX phones.

- Outbound local N11 (i.e. 411, 911) calls from the Asterisk IP PBX phones.

# How to Prepare for IP Flex Reach Service Turn-up

Preparation is crucial to a timely and successful implementation of IP Flex Reach. AT&T expects that the customer or their designated representative is knowledgeable in Asterisk administration. The Asterisk software should have been installed and properly operating prior to the circuit turn-up.

## Information from Customer to AT&T

AT&T will interview the customer prior to the installation. In addition to the normal logistics, the Asterisk administrator should be able to provide:

- IP address of the Asterisk server. Be sure to tell AT&T the address of the IP interface on the Asterisk server that is designated to be connected to the AT&T managed router. This address is called the signaling address in AT&T terms.

Note: The customer can get an AT&T IP address (begins with 12.) for their Asterisk or they can use one of their own.

- IP address range of any IP devices (e.g. SIP phones, ATAs, etc.) expected to send RTP directly to the AT&T network. (This means that you will have “canreinvite=yes” in profiles for these devices.) If you plan to run the RTP through the Asterisk server (not recommended), these address ranges are not needed since the media IP address is the Asterisk server’s IP address.

The customer should evaluate their desired dial plan. Two types of Direct Inward Dial (DIDs) numbers are available with AT&T IP Flex Reach: virtual Telephone Numbers (TNs) or non-virtual TNs.

- A virtual TN has an NPA that is different from the NPA nominally associated with the terminating customer site. In this case, AT&T will pass 10 digits to the Asterisk. For example, if an Asterisk associated telephone is assigned a virtual TN, the number received from AT&T would be 10 digits (e.g. 732-368-0451).

- A non-virtual TN has same NPA as nominally associated customer site. For a non virtual TN, AT&T will pass the length of the phone extension plus some prefix (called steering digits) if needed (typically a 4 digit extension without a prefix). If an Asterisk telephone is associated with a non virtual TN, the number received from AT&T would be 4 digits (i.e. 0451 for a 732-368-4051 TN).

The customer should communicate their dial plan requirements to AT&T so that the proper numbers are assigned.

## Information to Customer from AT&T

Sometime between the original interview and the turn-up, the customer should receive an email from AT&T containing the IP addresses of two IP Border Elements, commonly called “I-P-B-Es”. (An IPBE is called a Session Border Controller (SBC) in the general literature.)These addresses should of the form 12.x.y.z where x,y,z represent the rest of the IP address.  These addresses need to be included in your sip.conf configuration. Be sure to note the city associated with each address as a comment in your sip.conf file. A successful implementation is not possible without these addresses.

AT&T will provide the customer with the actual set of Direct Inward Dial (DID) numbers, prior to turn-up. These numbers must be assigned in the Asterisk’s extensions.conf file, prior to service turn-up. Be sure to confirm the number of digits to be received from the AT&T network.

AT&T expects the customer to prepend a 1 (send 1+10 digits) to the AT&T network for domestic calls. The code “011” should be prepended (send 011 + digits) for international calls.

Originating calls to AT&T must have the calling party number (CPN) set to a 10 digit number regardless of the type of TN associated with the originating phone. This 10 digit AT&T TN is nominally set as part of the “callerid” parameter in the Asterisk’s sip.conf file.

# Configuration Guide

This configuration guide specifies the Asterisk configuration files that are modified to support the AT&T IP Flexible Reach SIP trunking. AT&T expects that the customer or their designated representative is knowledgeable in Asterisk administration. The Asterisk software should have been installed and properly operating prior to the circuit turn-up. AT&T will NOT provide information or guidance on any Asterisk programming not related to SIP trunking.

The configuration files for SIP trunk programming are nominally found in the /etc/asterisk/ directory on the Asterisk server. (If the RealTime configuration is used, many of these variables are found in the database.) Configuration modifications are performed using a text editor, vi, in this case.

The database configuration was not tested.

System control, status monitoring and high level debugging is performed through the Command Line Interface (CLI). The CLI is invoked by entering:

asterisk –vvvvrgc at the root Linux command prompt. The results are displayed as follows:

[root@thorium ~]# asterisk –vvvvrgc

Asterisk 1.4.15, Copyright (C) 1999 - 2007 Digium, Inc. and others.

Created by Mark Spencer <markster@digium.com>

Asterisk comes with ABSOLUTELY NO WARRANTY; type 'core show warranty' for details.

This is free software, with components licensed under the GNU General Public

License version 2 and other licenses; you are welcome to redistribute it under

certain conditions. Type 'core show license' for details.

=========================================================================

== Parsing '/etc/asterisk/asterisk.conf': Found

== Parsing '/etc/asterisk/extconfig.conf': Found

Connected to Asterisk 1.4.15 currently running on thorium (pid = 32591)

Verbosity is at least 4

thorium*CLI>

Having the CLI open during service turn-up is very useful for diagnosing most common problems.

## Asterisk Version and Feature Requirements

The Asterisk Version 1.4.15 is certified. The underlying Linux Operating System was CentOS 4.4 SMP with the latest patches applied.

The Asterisk software version can be verified by running the show version command from the CLI. The results are displayed as follows:

thorium*CLI> core show version

Asterisk 1.4.15 built by root @ thorium on a i686 running Linux on 2007-12-18 14:19:15 UTC

thorium*CLI>

The Asterisk CLI also provides a debugging interface, which is invoked by entering:

thorium*CLI> sip set debug

SIP Debugging enabled

The debugging is disabled by entering:

thorium*CLI> sip set debug off

SIP Debugging Disabled

thorium*CLI>

In general, the SIP debugging mode should be off.

The G.729A codec licenses are not included with the Asterisk package. These licenses can be purchased separately, through the Digium, Inc. web site, www.digium.com . Note that installation of G.729A licenses requires that the server have access to the Internet. After the licenses are installed, the server can be moved to a private address space without Internet access.

To status the G.729A codecs, enter show g729 at the CLI. The response should be similar to the following:

thorium*CLI> show g729

0/0 encoders/decoders of 5 licensed channels are currently in use

thorium*CLI>

This case show 5 licensed channels with none currently in use.

## Asterisk Configuration Files

Asterisk configuration was performed with a series of “flat” files in the /etc/asterisk/ directory. Note: “;” in the first column is used to designate a “comment” line. Significant settings are highlighted with yellow background.

The significant files for configuring Asterisk for IP Flex Reach are:

- extensions.conf – sets dial plan

- rtp.conf – sets RTP port numbers to be used

- sip.conf – configures various SIP devices and AT&T’s IPBEs

Some additional configuration files are included in the appendices for reference. These are:

- zaptel.conf – Configures TDM PSTN basic DS1 parameters – Appendix A

- zapata.conf – Configures TDM PSTN Q.921 and Q.931 PRI parameters (located in the /etc directory) – Appendix A

- indications.conf – Sets US indications – Appendix B

- modules.conf – Controls loading of functional modules – Appendix B

Asterisk “contexts” are not specifically covered although some examples are included below. Be careful to include your contexts appropriately. Failure to do so may prevent successful call completion.

### extensions.conf

extensions.conf is used to set the dialing plan and all call routing. Call restrictions and calling capabilities are assigned in this file. Configuration shown here assigns the IPBEs to the variables SIPTRUNK1 and SIPTRUNK2. These variables are applied in the international, long distance, local and toll-free dialing commands.

For reference, an auto-attendant, voicemail retrieval, as well as the tested dial commands for the phones are included. Note that the extensions (four digit example for station-to-station dialing) and 10 digit numbers (DIDs) are specifically addressed. If the dialing plan features typical of IP Phones are used, this arrangement can be collapsed to the 10 digit only numbers.

The following example uses “mt_thorium” as the context for the SIP phones. Three IP phones (polycom, polycom2 and snom) are configured as extensions 0451, 0452 and 0453 respectively. A single Mediatrix ATA (device 8177) is assigned to 732 368-0454. The voicemail retrieval extension is (732) 368-0455 while the auto-attendant number is (732) 368-0450.

Only a portion of the extensions.conf file is illustrated here.

; extensions.conf - the Asterisk dial plan

;

;

[globals]

;

SIPTRUNK1=SIP/first_gateway			; SIP IPBE #1 Trunk interface

SIPTRUNK2=SIP/second_gateway		; SIP IPBE #2 Trunk interface

ISDNTRUNK=Zap/G26				;Trunk interface

;

TRUNKMSD=0				; MSD digits to strip (usually 1 or 0)

[trunkint]

;

; International long distance through trunk

;

exten => _011.,1,Dial(${SIPTRUNK1}/${EXTEN:${TRUNKMSD}})

exten => _011.,2,Dial(${SIPTRUNK2}/${EXTEN:${TRUNKMSD}})

[trunkld]

;

; Long distance context accessed through trunk

;

exten => _1NXXNXXXXXX,1,Dial(${SIPTRUNK1}/${EXTEN:${TRUNKMSD}})

exten => _1NXXNXXXXXX,2,Dial(${SIPTRUNK2}/${EXTEN:${TRUNKMSD}})

exten => _1NXXNXXXXXX,3,Dial(${ISDNTRUNK}/${EXTEN:0},30)

[trunklocal]

;

; Local seven-digit dialing for the 732 area code accessed through trunk interface

;

exten => _NXXXXXX,1,Dial(${SIPTRUNK1}/1732${EXTEN:${TRUNKMSD}})

exten => _NXXXXXX,2,Dial(${SIPTRUNK2}/1732${EXTEN:${TRUNKMSD}})

[trunktollfree]

;

; Long distance context accessed through trunk interface

;

exten => _1800NXXXXXX,1,Dial(${SIPTRUNK1}/${EXTEN:${TRUNKMSD}})

exten => _1800NXXXXXX,2,Dial(${SIPTRUNK2}/${EXTEN:${TRUNKMSD}})

exten => _1888NXXXXXX,1,Dial(${SIPTRUNK1}/${EXTEN:${TRUNKMSD}})

exten => _1888NXXXXXX,2,Dial(${SIPTRUNK2}/${EXTEN:${TRUNKMSD}})

exten => _1877NXXXXXX,1,Dial(${SIPTRUNK1}/${EXTEN:${TRUNKMSD}})

exten => _1877NXXXXXX,2,Dial(${SIPTRUNK2}/${EXTEN:${TRUNKMSD}})

exten => _1866NXXXXXX,1,Dial(${SIPTRUNK1}/${EXTEN:${TRUNKMSD}})

exten => _1866NXXXXXX,2,Dial(${SIPTRUNK2}/${EXTEN:${TRUNKMSD}})

[privatetrunkld]

;

; Long distance context accessed anonymously through trunk

;

exten => _11NXXNXXXXXX,1,SIPAddHeader(P-Preferred-Identity:"${CALLERID(name)}"<sip\:${CALLERID(num)}\@172.16.4.4\:5060\>)

exten => _11NXXNXXXXXX,n,SIPAddHeader(Privacy: id)

exten => _11NXXNXXXXXX,n,SetCallerID(Anonymous <restricted>,a)

exten => _11NXXNXXXXXX,n,Dial(${TRUNK}/${EXTEN:1})

;

; Arrange contexts as needed

;

[international]

;

; Master context for international long distance

;

include => longdistance

include => trunkint

[longdistance]

;

; Master context for long distance

;

include => local

include => trunkld

include => privatetrunkld

[local]

;

; Master context for local, toll-free, and stations

;

include => trunklocal

include => mt_thorium

include => trunktollfree

include => pri_from_als

[mainmenuVOIP]

include => mt_thorium               ; Note include of SIP phone context

;

; Test VOIP ATT Auto-attendant - Auto Attendant "main menu" context with submenus

;

exten => 7323680450,1,Answer

exten => 7323680450,2,Background(/var/lib/asterisk/sounds/directory)

exten => 7323680450,3,goto(Auto-Attendant,startwork,1)

exten => i,1,Hangup

[Auto-Attendant]

exten => startwork,1,Background(/var/lib/asterisk/sounds/enter-ext-of-person)     ;

exten => startwork,2,WaitExten

exten => 1,1,Macro(stdexten,0451,SIP/polycom)                    ; "Call polycom"

exten => 2,1,Macro(stdexten,0452,SIP/polycom2)		  ; "Call polycom2"

exten => 3,1,Macro(stdexten,0453,SIP/snom)    		  ;"Call snom"

exten => 4,1,Meetme,8600|M

exten => 5,1,Directory(mt_thorium)

exten => t,1,Dial(SIP/polycom,30,r)                                ; "Call default"

exten => i,1,Background,/var/lib/asterisk/sounds/num-not-in-db

exten => i,2,Hangup

[mt_thorium]

include => longdistance

include => mainmenuVOIP

include => local

exten => 7323680455,1,VoicemailMain()             ; General Voicemail pickup

exten => 7323680455,2,Hangup

exten => 0455,1,VoicemailMain(()@mt_thorium)       ; General Voicemail pickup

; 								; External DID

exten => 0455,2,Hangup

;

; Add local extensions to dial plan

;

exten => 0451,1,Macro(stdexten,0451,SIP/polycom)                    ; Polycom 600

exten => 0452,1,Macro(stdexten,0452,SIP/polycom2)                  ; Polycom 600

exten => 0453,1,Macro(stdexten,0453,SIP/snom)                        ; Polycom 600

;

; Add external numbers to dial plan

;

exten => 7323680451,1,Macro(stdexten,0451,SIP/polycom)         ; Polycom 600

exten => 7323680452,1,Macro(stdexten,0452,SIP/polycom2)       ; Polycom 600

exten => 7323680453,1,Macro(stdexten,0453,SIP/snom)

exten => 7323680454,1,Dial(SIP/8177,30)                                ; "FAX Line "

### rtp.conf

This rtp.conf file has been modified to include the rtp ports used in the AT&T service.

;

; RTP Configuration

;

[general]

;

; RTP start and RTP end configure start and end addresses

;

rtpstart=16384

rtpend=32767

;

; Whether to enable or disable UDP checksums on RTP traffic

;

;rtpchecksums=no

;

; The amount of time a DTMF digit with no 'end' marker should be

; allowed to continue (in 'samples', 1/8000 of a second)

;

;dtmftimeout=3000

### sip.conf

This file assigns the IPBEs to variables (first_gateway and second_gateway) and assigns a SIP PING (NOTIFY Message) to be sent on a regular interval (2000 msec in the example). Other data, like the IP address assignment, is shown as well. Only an abbreviated version of the sip.conf file is provided here.

sip.conf

;

; SIP Configuration example for Asterisk with AT&T services

;

[general]

context=pri_from_als        ; Default context for incoming calls

allowoverlap=yes

realm=asterisk                 ; Realm for digest authentication

; defaults to "asterisk"

; Realms MUST be globally unique according to

; RFC 3261

; Set this to your host name or domain name

bindport=5060                 ; UDP Port to bind to (SIP standard port is 5060)

bindaddr=172.16.4.4        ; IP address to bind to (0.0.0.0 binds to all)

srvlookup=no                  ; Enable DNS SRV lookups on outbound calls

; Note: Asterisk only uses the first host

; in SRV records

; Disabling DNS SRV lookups disables the

; ability to place SIP calls based on domain

; names to some other SIP users on the Internet

videosupport=no              ; Turn on support for SIP video

disallow=all                        ; First disallow all codecs

allow=g729                        ; Allow codecs in order of preference

allow=ulaw

;

musicclass=sip                   ; Sets the default music on hold class for all SIP calls

; This may also be set for individual users/peers

language=en                      ; Default language setting for all users/peers

; This may also be set for individual users/peers

sendrpid = no                   ; If Remote-Party-ID should be sent

useragent=Thorium Asterisk PBX   ; Allows you to change the user agent string

usereqphone = yes             ; If yes, ";user=phone" is added to uri that contains

; a valid phone number

dtmfmode = rfc2833          ; Set default dtmfmode for sending DTMF. Default.

;

;--------------------------------- T.38 FAX PASSTHROUGH SUPPORT --------------------

;

;This setting is available here, the [general] section, and in device configurations.

; Setting this to yes, enables T.38 fax (UDPTL) passthrough on SIP to SIP calls, if

; both parties have T38 support enabled in their Asterisk configuration

; This has to be enabled in the general section for all devices to work. You can

; then disable it on a per device basis.

;

; T.38 faxing only works in SIP to SIP calls, with no local or agent channel being used.

;

t38pt_udptl = no            ; T.38 not officially supported

t38pt_rtp=no

t38pt_tcp=no

;

;----------------------------------- MEDIA HANDLING --------------------------------

; By default, Asterisk tries to re-invite the audio to an optimal path. If there's

; no reason for Asterisk to stay in the media path, the media will be redirected.

; This does not really work with in the case where Asterisk is outside and have

; clients on the inside of a NAT. In that case, you want to set canreinvite=nonat

;

canreinvite=yes         ; Asterisk by default tries to redirect the

; RTP media stream (audio) to go directly from

; the caller to the callee.  Some devices do not

; support this (especially if one of them is behind a NAT).

; The default setting is YES. If you have all clients

; behind a NAT, or for some other reason wants Asterisk to

; stay in the audio path, you may want to turn this off.

; In Asterisk 1.4 this setting also affect direct RTP

; at call setup (a new feature in 1.4 - setting up the

; call directly between the endpoints instead of sending

; a re-INVITE).

directrtpsetup=yes  ; Enable the new experimental direct RTP setup. This sets up

; the call directly with media peer-2-peer without re-invites.

; Will not work for video and cases where the callee sends

; RTP payloads and fmtp headers in the 200 OK that does

;not match the callers INVITE.

;

; Heartbeat to AT&T gateways, one entry per IPBE

;

[first_gateway]

type=peer

host=135.25.29.74                 ; First AT&T IP Border Element (IPBE) Address

qualify=2000                         ; SIP Ping Frequency in msec.

;

[second_gateway]

type=peer

host=135.25.29.1                  ; Second AT&T IP Border Element (IPBE) Address

qualify=2000                         ; SIP Ping Frequency in msec.

;

Note that these hosts are unsecured. All access to these IPBEs is strictly limited to assigned AT&T subscribers.

******** NOTE IP Addresses above are only EXAMPLES! An AT&T representative will provide the correct addresses for your circuit. ***********

# Troubleshooting

This section provides some tips about troubleshooting problems

- Asterisk Command Line Interface (CLI)

The Asterisk Command Line Interface (CLI) is used to view the call flow as well as provide an overall debugging interface.  From a terminal screen, logged in as root, enter asterisk –vvvvrgc  General debugging should be performed from this interface. Typing a “?” at the CLI prompt will show all commands. The TAB key can be used to complete the partial entry of any command

- SIP debugging - sip debug

Detailed SIP message debugging is enabled at the CLI with the command:

sip debug . The command: sip no debug will disable the debugging.

- Status of SIP peers and friends

The status of all peers can be show via the CLI with the command:

sip show peers

Some of the typical problems include:

- None of my stations are registered – Enable the sip debug as shown above.

  - If REGISTER messages are reaching the Asterisk, correct the device configuration or Asterisk peer profile.

  - If REGISTER messages are not reaching the Asterisk, check to see if the iptables and ip6tables are disabled and the services are not running. These firewalls may be enabled later and configured to pass the appropriate SIP / RTP traffic.

- Calls from the AT&T Network aren’t reaching the Asterisk - Check to see if the iptables and ip6tables are disabled and the services are not running. These firewalls may be enabled later and set to pass the appropriate SIP / RTP traffic.

- Calls seem to connect, but there is no audio – Set the parameters canreinvite = no and directrtpsetup = no in sip.conf. Reload the Asterisk to make these settings active. This will force the RTP through the Asterisk server. Place another call. If the audio path is successfully established, an issue with NAT may exist within the customer’s network.

- Incoming calls don’t a ring device but I see it fail on the CLI – Review the CLI output carefully. Check to see that the called digits exactly match the extension of a device that is registered with your Asterisk. If the digits match, carefully check your context “include” statements.

- Poor voice quality – Check the Asterisk’s ethernet interface settings to verify that they are at least 100 Mb, Full Duplex.

Unable heartbeat (SIP Options) with IPBEs – If the ABE Server has multiple ethernet interfaces, static routes will be required to insure that traffic for the IPBE is directed to the correct interface. These routes can be added at the Linux command line. Once the routes are working, adding them permanently can be done in the /etc/rc.local Linux file.

A good reference for general Asterisk information and programming is:

Van Meggelen, J., Madsen, L. and Smith, J.  Asterisk, The Future of Telephony, 2nd Edition,  O’Reilly Media, Inc 2007 ISBN-10: 0-596-51048-9, ISBN-13: 978-0-596-51048-0

This Customer Configuration Guide ("CCG") is offered as a convenience to AT&T's customers.  The specifications and information regarding the product in this CCG are subject to change without notice.  All statements, information, and recommendations in this CCG are believed to be accurate but are presented without warranty of any kind, express or implied, and are provided “AS IS”.  Users must take full responsibility for the application of the specifications and information in this CCG.

APPENDICES

These appendices address several aspects of the Asterisk configuration that are not directly related to configuring for the AT&T IP-Flex Reach service. An Asterisk administrator maybe asked to include these capabilities. These files are provided as working examples.

# Appendix - PSTN / Primary Rate Interface (PRI) Overflow

The Asterisk server can be outfitted with analog or digital interfaces to the traditional PSTN. The configuration files, /etc/zaptel.conf and /etc/asterisk/zapata.conf, set the appropriate parameters.

This particular configuration provides for a three T1 Non Facility Associated Signaling (NFAS) Primary Rate Interface (PRI). This PRI has a primary D-channel in channel 24 on the first T1 (Facility ID 0) and a backup D-Channel in channel 48 (on the second T1 (Facility ID 1). The third T1 does not contain a D-channel. All channels are arranged in trunkgroup labeled “26”. Note that the D-Channel configuration and Facility ID were selected to interface to a 4ESS. Assignment of the trunkgroup number is arbitrary.

## zaptel.conf

This configuration file sets the physical layer parameters (Layer 1) associated with any TDM hardware in the Asterisk server. The file is nominally located in the /etc directory.

The values below are only those pertinent to the configuration of PRI interfaces in the /etc/zaptel.conf. Additional commentary and parameters will be contained in the actual file.

#

# Zaptel Configuration File

#

# This file is parsed by the Zaptel Configurator, ztcfg

#

#

# First come the span definitions, in the format

#

# span=<span num>,<timing source>,<line build out>,<framing>,<coding>[,yellow]

#

span=1,1,0,esf,b8zs

span=2,0,0,esf,b8zs

span=3,0,0,esf,b8zs

#

#

# Next come the definitions for using the channels.  The format is:

# <device>=<channel list>

#

# The channel list is a comma-separated list of channels or ranges

#

bchan=1-23

dchan=24

bchan=25-27,49

dchan=48

#

# Finally, you can preload some tone zones

#

# You may also specify a default zone with "defaultzone=<zone>" where zone

# is a two letter country code.

#

loadzone = us

defaultzone=us

#

## zapata.conf

This configuration file sets the link and signaling layer parameters (Layers 2 and 3) associated with the PRI hardware in the Asterisk server. The file is nominally located in the /etc/asterisk directory.

The values below are only those pertinent to the configuration of PRI interfaces in the /etc/asterisk/zapata.conf. Additional commentary and parameters will be contained in the actual file.

;

; Zapata telephony interface

;

; Configuration file

;

[trunkgroups]

;

; Trunk groups are used for NFAS or GR-303 connections.

;

; Group: Defines a trunk group.

;        group => <trunkgroup>,<dchannel>[,<backupdchannel>...]

;

trunkgroup => 26,24,48

;

; Spanmap: Associates a span with a trunk group

;        spanmap => <zapspan>,<trunkgroup>[,<interfaceID>]

;

spanmap => 1,26,0

spanmap => 2,26,1

spanmap => 3,26,2

[channels]

;

; Default language

;

language=en

;

; Default context

;

context=pri_from_als

;

; Switchtype:  Only used for PRI.

;

;

switchtype=4ess

;;

; PRI Dialplan:  Only RARELY used for PRI.

;;

pridialplan=national

;

; PRI Local Dialplan:  Only RARELY used for PRI (sets the calling number's numbering plan)

;

prilocaldialplan=national

;

; PRI resetinterval: sets the time in seconds between restart of unused

; channels, defaults to 3600

;

resetinterval = 14400

;

; PRI Out of band indications.

; Enable this to report Busy and Congestion on a PRI using out-of-band

; notification

;

;

priindication = outofband

;

; If you need to override the existing channels selection routine and force all

; PRI channels to be marked as exclusively selected, set this to yes.

priexclusive = yes

;

; ISDN Timers

; All of the ISDN timers and counters that are used are configurable.

;

pritimer => t200,1000

pritimer => t313,4000

;

; To enable transmission of facility-based ISDN supplementary services (such

; as caller name from CPE over facility), enable this option.

facilityenable = yes

;

; Signalling method (default is fxs).  Valid values:

; pri_cpe:        PRI signalling, CPE side

; pri_net:        PRI signalling, Network side

;

;

; Whether or not to use caller ID

;

usecallerid=yes

;

; Whether or not use the caller ID presentation for the outgoing call that the

; calling switch is sending.

;

usecallingpres=yes

;

; Support Caller*ID on Call Waiting

;

callwaitingcallerid=yes

;

; Support three-way calling

;

threewaycalling=yes

;

; Support flash-hook call transfer (requires three way calling)

; Also enables call parking (overrides the 'canpark' parameter)

;

transfer=yes

;

; Allow call parking

; ('canpark=no' is overridden by 'transfer=yes')

;

canpark=yes

;

; Support call forward variable

;

cancallforward=yes

;

; Whether or not to support Call Return (*69)

;

callreturn=yes

; Enable echo cancellation

; Use either "yes", "no", or a power of two from 32 to 256 if you wish to

; actually set the number of taps of cancellation.

;

echocancel=yes

;

; Generally, it is not necessary (and in fact undesirable) to echo cancel when

; the circuit path is entirely TDM.  You may, however, reverse this behavior

; by enabling the echo cancel during pure TDM bridging below.

;

echocancelwhenbridged=yes

;

; In some cases, the echo canceller doesn't train quickly enough and there

; is echo at the beginning of the call.  Enabling echo training will cause

; asterisk to briefly mute the channel, send an impulse, and use the impulse

; response to pre-train the echo canceller so it can start out with a much

; closer idea of the actual echo.  Value may be "yes", "no", or a number of

; milliseconds to delay before training (default = 400)

;

echotraining=yes

;echotraining=800

; Logical groups can be assigned to allow outgoing rollover.  Groups range

; from 0 to 63, and multiple groups can be specified.

;

group=26

;

; CallerID can be set to "asreceived" or a specific number if you want to

; override it.

;

callerid=asreceived

;

; Few zones are supported at the time of this writing, but may be selected

; with "progzone"

;

; This feature can also easily detect false hangups. The symptoms of this is

; being disconnected in the middle of a call for no reason.

;

callprogress=yes

progzone=us

;

; For fax detection, uncomment one of the following lines.  The default is *OFF*

;

faxdetect=both

;faxdetect=incoming

;faxdetect=outgoing

;faxdetect=no

;

; This option specifies a preference for which music on hold.

;

mohinterpret=sip

;

; This option specifies which music on hold class to suggest to the peer channel

;

mohsuggest=sip

;

; Sample PRI (CPE) config:  Specify the switchtype, the signalling as either

; pri_cpe or pri_net for CPE or Network termination, and generally you will

; want to create a single "group" for all channels of the PRI.

;

signalling = pri_cpe

channel => 1-23,25-27,49

# Appendix - Other Asterisk Configuration Files

These configuration files are provided for completeness.

## indications.conf

This indications.conf file is configured to support US locations. All other countries have been removed.

;

; indications.conf

; Configuration file for location specific tone indications

; used by the pbx_indications module.

;

[general]

country=us		; default location

;

[us]

description = United States / North America

ringcadance = 2000,4000

dial = 350+440

busy = 480+620/500,0/500

ring = 440+480/2000,0/4000

congestion = 480+620/250,0/250

callwaiting = 440/300,0/10000

dialrecall = !350+440/100,!0/100,!350+440/100,!0/100,!350+440/100,!0/100,350+440

record = 1400/500,0/15000

info = !950/330,!1400/330,!1800/330,0

stutter = !350+440/100,!0/100,!350+440/100,!0/100,!350+440/100,!0/100,!350+440/100,!0/100,!350+440/100,!0/100,!350+440/100,!0/100,350+440

## modules.conf

The modules.conf file shown here is the one specifically used for AT&T testing. The settings described here do not directly affect the AT&T interoperability. This file’s content is included for completeness and may differ for a given installation.

;

; Asterisk configuration file

;

; Module Loader configuration file

;

[modules]

autoload=yes

;;

noload => pbx_gtkconsole.so

noload => pbx_kdeconsole.so

; Not using DUNDI

noload => pbx_dundi.so

;

; Intercom application is obsoleted by chan_oss.  Don't load it.

;

noload => app_intercom.so

;

; Load either OSS or ALSA, not both

; By default, load OSS only (automatically) and do not load ALSA

;

noload => chan_alsa.so

;

; Don't load MGCP, Skinny, ooh323 and iax2 because we are not using them

;

noload => chan_mgcp.so

noload => chan_skinny.so

noload => chan_ooh323.so

noload => chan_iax2.so

;

noload => app_enumlookup.so

;

noload => res_config_pgsql.so

noload => res_config_mysql.so

noload => res_config_odbc.so

noload => cdr_addon_mysql.so

noload => cdr_pgsql.so

noload => cdr_odbc.so

;

; Don't load unused codecs

;

noload => codec_alaw.so

noload => codec_speex.so

noload => codec_g726.so

noload => codec_adpcm.so

noload => codec_lpc10.so

;

# Appendix – Telephone Configuration in sip.conf

The Polycom 600, Snom 360 and the Mediatrix 1102 were explicitly tested. Their respective profiles in /etc/asterisk/sip.conf are included as examples.

Snom 360, Firmware 6.5.10

[snom]

type=friend

username=snom

secret=att

host=dynamic

context=mt_thorium

subscribecontext=mt_thorium     ; Only allow SUBSCRIBE for local extensions

nat=no

canreinvite=yes                  ; Asterisk by default tries to redirect the

videosupport=no

callerid=snom * * <7323860453>

qualify=200                     ; Consider it down if it's 1 second to reply

qualifysmoothing=yes

mailbox=0453@mt_thorium                ; Set MWI to extension, not device

callingpres=allowed_not_screened ; control presentation

dtmfmode=rfc2833                ; Choices are inband, rfc2833, or info

Polycom 600, Firmware 2.2.0.0047

[polycom]

type=friend

username=polycom                  ; Polycom 600

secret=att

callerid=polycom <7323680451>

context=mt_thorium

subscribecontext=mt_thorium     ; Only allow SUBSCRIBE for local extensions

host=dynamic

nat=no

canreinvite=yes                  ; Asterisk by default tries to redirect the

videosupport=no

qualify=200                     ; Qualify peer is no more than 200ms away

qualifysmoothing=yes

mailbox=0451@mt_thorium         ; Set MWI

callingpres=allowed_not_screened ; control presentation

dtmfmode=rfc2833                ; Choices are inband, rfc2833, or info

Mediatrix 1102, Firmware 5.0.22.138 E

[8177]

type=friend

username=8177

secret=att

callerid=mediatrix_01 <7323680454>

context=mt_thorium

subscribecontext=mt_thorium     ; Only allow SUBSCRIBE for local extensions

host=dynamic

nat=no

canreinvite=yes                  ; Asterisk by default tries to redirect the

t38pt_udptl=no                  ; Enable T.38 fax support

videosupport=no

qualify=300                     ; Consider it down if it's 1 second to reply

disallow=all

allow=ulaw

callingpres=allowed_not_screened ; control presentation

dtmfmode=rfc2833                ; Choices are inband, rfc2833, or info

[8178]

type=friend

username=8178

secret=att

callerid=mediatrix_02 <7323680454>

context=mt_thorium

subscribecontext=mt_thorium     ; Only allow SUBSCRIBE for local extensions

host=dynamic

nat=no

canreinvite=yes                  ; Asterisk by default tries to redirect the

t38pt_udptl=no                  ; Enable T.38 fax support

videosupport=no

qualify=300                     ; Consider it down if it's 1 second to reply

disallow=all

allow=ulaw

callingpres=allowed_not_screened ; control presentation

dtmfmode=rfc2833                ; Choices are inband, rfc2833, or info

# Appendix – Use of Other SIP Devices

One of the significant features of Asterisk is that a variety of phones and ATAs can be utilized. The Polycom 600 and Snom 360 were explicitly tested by AT&T in this guide. Use of other devices is done at the risk of the Asterisk administrator. Information regarding the experiences of other users can be found at:

http://www.voip-info.org/wiki-Asterisk+phones

Various VOIP phone vendors provide guides for using their equipment with Asterisk.  This material may provide guidance but there is no guarantee the device will work properly with the AT&T network.

# Appendix - G.711 and T.38 Analog Terminal Adapters

Several Analog Terminal Adapters (ATA) were tested with Asterisk 1.4.15. AT&T has had partial success with T.38 when using two of these devices. All devices tested successfully when G.711 was used for fax transmission.

- Mediatrix 1102 – An ATA using the SIP protocol to communicate to the Asterisk IP PBX for call feature and routing support. This ATA provides an endpoint for T.38 fax. This ATA was using firmware version 5.0.22.138 E.

- UTStarcom iAN-02EX – An analog terminal adaptor (ATA) using the SIP protocol to communicate to the Asterisk IP PBX for call feature and routing support. This ATA provides an endpoint for T.38 fax. This ATA was using firmware version V4.8.2.50b.  This device is now manufacturer discontinued.

- Sipura 3000 – An ATA using the SIP protocol to communicate to the Asterisk IP PBX for call feature and routing support. This device does not support T.38 fax protocol. This ATA was using firmware version 2.0.13 (GWg). This device is now manufacturer discontinued.